Problem 1 (Signal reconstruction after sampling):

Given the following signal

Task (a)

Implement a function ss2_problem1() that has one input parameter n which controls the sampling frequency. The function is required to generate a plot containing two subplots.

function[] = ss2_problem1(n)

x_given = linspace(0,1);

y_given = 4*sin(2*pi*x_given)+cos((pi/4)+16*pi*x_given);

xs = 0: n:1;

ys = 4*sin(2*pi*xs)+cos((pi/4)+16*pi*xs);

subplot(2,1,1)

plot(x_given,y_given);

subplot(2,1,2);

stem(xs,ys);

xlabel(['Sampling Frequency Factor ',num2str(n),''])

end

Task(b)

In the first subplot, plot the signal x ( t ) within the interval t ∈ [0.0025, 1] using a temporal resolution of T = 0.0025.

function[] = ss2_problem1(n)

x_given = 0.0025:0.0025:1;

y_given = 4*sin(2*pi*x_given)+cos((pi/4)+16*pi*x_given);

xs = 0.0025: n:1;

ys = 4*sin(2*pi*xs)+cos((pi/4)+16*pi*xs);

subplot(2,1,1)

plot(x_given,y_given);

subplot(2,1,2);

stem(xs,ys);

xlabel(['Sampling Frequency Factor ',num2str(n),''])

end

Task(c)

Sample the signal x(t) and plot the sampled values xk in the first subplot, without deleting the previous plot. The sampling interval Ts is specified by Ts= T · n. That is, the larger the value of n, the lower the sampling frequency. Please ensure that the plots share the same vertical axis (identical scaling) to enable a direct comparison. Hint: The Signal Processing Toolbox provides the helpful function downsample().

function[] = ss2_problem1(n)

x_given = .0025:0.0025:1;

y_given = 4*sin(2*pi*x_given)+cos((pi/4)+16*pi*x_given);

plot(x_given,y_given);

hold on

xs = 0:n:1;

ys = 4*sin(2*pi*xs)+cos((pi/4)+16*pi*xs);

stem(xs,ys);

xlabel(['Sampling Factor ',num2str(n),''])

hold off

end

Task(d)

Test your MATLAB function by varying the values of the input parameter n. The sampled values show when the sampling theorem is met/ not met. This limit of n is also to be found using theoretical considerations.

We tested the function with different inputs and it seems that the sampling theorem is met.

Task(e)

If the sampling theorem is met, the original signal x(t) can be reconstructed from the sampling values x

k(t) based on the following relationship

You can use the MATLAB function sinc() for this purpose (check its definition!). Plot the reconstructed signal ˜x(t) in the second subplot. Hint: To verify your code, you may want to plot the individual summands as well as the original signal x (t).

function[] = SS_lab1_e(T)

x=0:0.05:2; %add 20 past and future samples

y=4*sin(2*pi*x)+cos((pi/4)*16*pi*x);

x1 = 0:0.05:2;

y1=4*sin(2*pi*x1)+cos((pi/4)*16*pi*x1);

for l=1:200 %observed interval is [0,0.02]

t=(l­1)*T/100;%successive sample separation is 0.01T

h=sinc(pi*(t­x1.*T)./T);

xr(l)=y1*h.'; %approximate interpolation of (4.11)

end

subplot(2,1,1);

hold on

plot(x,y);

stem(x1,y1);

xlabel(['Sampling Period is ',num2str(T),''])

hold off

subplot(2,1,2);

plot(xr);

xlabel(['Sampling Period is ',num2str(T),''])

end

Task(f)

If the sampling theorem is satisfied, you should observe a close match between the reconstructed signal and the original signal. Also plot the reconstructed signal for a case where the sampling theorem is violated.